Technical  Data :

| Product Name : | High Precision Mini CNC Milling Machine |
| --- | --- |
| Product Item : | MM-VM300 |
| Control System : | PC-based MACH3 CNC Control System |
| Working Area |  |
| Table dimensions : | 600 X 190 mm |
| Table Load capacity : | 150 KGS |
| Number of T-slots : | 3 pieces |
| T-slots, width : | 12 mm |
| T-slots, spacing : | 49 mm |
| Spindle nose-to-table distance : | 130—350 mm |
| Throat : | 235 mm |
| Travels |  |
| Travel X-Axis : | 300 mm |
| Travel Y-Axis : | 180 mm |
| Travel Z-Axis : | 290 mm |
| Headstock |  |
| Spindle Speed : | 100-6000 RPM |
| Spindle Taper : | MT3 / BT30 |
| Max. Moving Speed : | 8000 mm /min |
| The Max. Feeding Speed : | 4000 mm /min |
| Position Accuracy : | ±0.02 mm |
| Repeatability Accuracy : | ±0.01 mm |
| Driving Capacity |  |
| Main Motor : | 1100W / 1500W Powerful Motor |
| X-Axis Motor : | 4 NM |
| Y-Axis Motor : | 4 NM |
| Z-Axis Motor : | 6 NM |
| Max Spindle Motor Torque : | 5.5 NM |
| Coolant Pump Motor: | 400W |
| Coolant Tank Capacity : | 29 Liter |
| Connecting Voltage : | Single Phase 240V or Three Phase 400V |
| Measures and Weights |  |
| Total Driving Motor : | 4000W |
| Gross Weight : | 450 KGS |
| Packing Size : | 1150 X 1100 X 2000 mm |
| Standard Accessory : | MACH3 CNC Control System, Lubrication Equipment Coolant Pump, Oil Tray, Tools and Tool Box |