KÝ HIỆU VIẾT TẮT :

| UART8250 | Universal Asynchorous Receive Transmitte 8250 |  |
| --- | --- | --- |
| Sint(11) | Serial int (stream int) | I/OR : Input/Output Read |
| Sout(10) | Serial out (stream out) | I/OW Input/Oput Write |
| DTR(33) | Data Terminal Ready | DOST : Dostrobe |
| DSR(37) | Data Set Ready | INTR : Interrupt |
| RTS(32) | Request to sent | MR : Master Reset |
| CTS(36) | Clear to sent | ADS : Address strobe |
| CD | Carrier Detect | TTL : Transistor to Transistor logic |
| RI(39) | Ring Indicator | CS : Chip Select |
| THR | Transermitted Holding Register | Crystal(Xtal) |
| TSR | Transermitted Shift Register | Baudout |
| RSR | Receive Shift Register | RCCK : Receive Clock |
| RBR(3F8h) | Receive Buffer Register |  |
| LCR(3FB) | Line Control Register |  |
| Divisor |  |  |
| LSR | Line Status Register |  |
| MCR(3FCh) | Modem Control Register |  |
| MSR(3FEh) | Modem Status Register |  |
| IER | Interrupt Enalbe Rigister |  |
| IIR | Interrupt Identifier Reg |  |

Line Control Register: 3FB

| B7 | B6 | B5 | B4 | B3 | B2 | B1 | B0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 1= 0=mode | Set break | Stick parity | Even Parity | Enable parity | Stop bit | 0 | 0 |
| Devisor = 1843200/(Baud*16) B2 : 0 ( 1 stop bit) B2 : 1 (1 ½ or 2 stop bit) B3 : Enable parity B4: Even parity B1B0: 00 (5 bit data) B1B0: 01(6 bit data) B1B0: 10(7 bit data) B1B0: 11(8 bit data) |  |  |  |  |  | 0 | 1 |
|  |  |  |  |  |  | 1 | 0 |
|  |  |  |  |  |  | 1 | 1 |